# Macro-Star Networks: Efficient Low-Degree Alternatives to Star Graphs

Chi-Hsiang Yeh, Emmanouel A. Varvarigos

IEEE Transactions on Parallel and Distributed Systems, Vol.9, No. 10, October 1998, p987-1003

Abstract

We propose a new class of interconnection networks, called macro-star networks, which belong to the class of Cayley graphs and use the star graph as a basic building module.

- Introduction

The purpose of this paper is to develop a new family of parallel architectures that meet the following requirements:

- small node degree,

- small diameter,

- symmetry properties,

- efficient emulation of popular topologies,

- balanced traffic, and

- suitability for VLSI implementation.

An MS(l,n) network has N = (nl + 1)! nodes, degree n+l–1.

Balanced MS networks: An MS(l, n) network with l=Θ(n) is asymptotically within a factor of 1.25 from a universal lower bound.

- Macro-Star Networks

- Routing and Topological Properties

3.1 Routing in an MS Network Based on Any Star-Graph Routing Algorithm

Any Algorithm in an (ln +1)-star under the Single-Dimension Communication (SDC) model can be emulated on the MS(l,n) network with a slowdown factor of three.

3.2 Faster MS Routing Algorithms

The total number of steps required by the routing algorithm is at most equal to 2.5(nl+l−1) steps.

3.3 Basic Properties

3.4 Near Optimal Diameter

- Parallel Algorithms in MS Networks

4.1 Parallel Algorithms under the Single-Dimension Communication Model

4.2 Emulation of Star Graphs under the All-Port Communication Model

4.3 Embeddings of Trees, Meshes, Hypercubes, and Complete Transposition Graphs

- Conclusions

The MS network and its variant topologies can fit the needs of high-performance interconnection networks and appear to be efficient low-degree alternatives to the star graph for building medium to large-scale parallel architectures.

Created by: Chia-Mao Huang

Date: June 22, 2000