Connective Fault Tolerance in Multiple-Bus Systems

Hung-Kuei Ku, John P. Hayes

IEEE Transactions on Parallel and Distributed Systems, Vol. 8, No. 6, June 1997, pp. 574-585

1. Introduction

We propose a graph model for multiple-bus systems which can represent processor, bus , and link (partial bus).

2. Connectivity and Faults

2.1 Preliminaries

The PBL graph of Fig. 2a is (2,1,1)-FT.

{b1, b2} is a minimum critical bus-fault set in the BPL graph of Fig. 2a.

2.2 Processor Fault Tolerance

P-nodes are connected in a PBL graph if and only if its PAG is connected.

G can tolerance kp processor faults if and only if Gp is (kp+1)-connected.

Theorem 1. A PBL graph G is (k(Gp)-1)-PFT where Gp is the PAG of G.

Corollary 1. The minimum critical processor-fault sets of a PBL graph G are the minimum node cuts of its PAG Gp.

The PBL graph of Fig. 3a is two-PFT because the connectivity of its PAG of Fig. 3b is three.  Fig. 4 is one-PFT and {p3, p5} is the only minimum critical processor-fault set.

2.3 Bus Fault Tolerance

Lemma 1. Let G by a PBL graph with no isolated P-node or B-node, and let G contain at least two P-nodes and two B-nodes.  The P-nodes of G are connected if and only if its B-node are connected.

bp(G) : the minimum neighborhood size of the P-nodes of a PBL graph G.

e.g. bp(G¢) = 2 for the PBL graph G¢ of Fig. 3a.

Theorem 2. Let G be a PBL graph that contains b B-nodes.  If bp(G) < b, then G is (min{k(Gb), bp(G)} - 1)-BFT, where Gb is the BAG of G; otherwise, G is (bp(G) - 1)-BFT.

Corollary 2. Let Gb be the BAG of a PBL graph G.  Then the minimum critical bus-fault sets of G are given by the following three cases:

- The minimum node cuts of Gb, if bp(G) < b and k(Gb) < bp(G);

- The minimum neighborhoods of the P-nodes of G, if bp(G) = b, k(Gb) > bp(G), or k(Gb) = bp(G) = b-1;

- All the minimum critical bus-fault sets described in the previous two cases, if k(Gb) = bp(G) < b - 1.

e.g. G¢ is one-BFT because k(Gb) = bp(G) = 2(b=3). Removing any two B-nodes from G¢ disconnects the P-nodes of G¢.  Gig.4a is also one-BFT because k(Gb) = bp(G) = 2(b=4).  The minimum critical bus-fault sets of G¢¢ are {b1, b2}, {b2, b3}, {b3, b4}.

2.4 Link Fault Tolerance

Lemma 2. The P-nodes of a PBL graph G with no isolated P-node are connected if and only if the edges of G are connected.

dp(G) : the minimum node degree of the P-node in G.

Theorem 3. A PBL graph G is (min{k(Gl), dp(G)} - 1)-LFT where Gl is the LAG of G.

Corollary 3. Let Gl be the LAG of a PBL graph G.  Then the minimum critical link-fault sets of G are given by the following three cases:

- The minimum node cuts of Gl, if k(Gl) < dp(G);

- Edge sets, each of which contains all the edges incident with a P-node of degree dp(G), if k(Gl) > dp(G);

- All the minimum critical link-fault sets described in the previous two cases, if k(Gl) = dp(G).

e.g. k(G¢l) = 3 in Fig. 3d.  Therefore, the PBL graph G¢ is one-LFT because dp(G¢) = 2.  The minimum critical link-fault sets are {l1, l2}, {l6, l7}, and {l8, l9}.  k(G¢¢l) = 2.  Its minimum node cuts are {l7, l10} and {l8, l12}.  Since dp(G¢¢) = 2, PBL graph G¢¢ is one-LFT, and its minimum critical link-fault sets are {l7, l10}, {l8, l12}, {l1, l2}, {l3, l4}, and {l8, l9}.

3. Applications

3.1 M-LANs(multichannel local area network)

Theorem 4. An M-LAB with p ³ 2 processors and b buses is (p-2, b-1, b-1)-FT.

3.2 Spanning-Bus Hypercubes

Lemma 6. A w-wide d-dimensional spanning-bus hypercube is (d - 1)-BFT.

Lemma 7. A w-wide d-dimensional spanning bus hypercube is ( d - 1)-LFT.

Created by: Kuo-Shii Huang

Date: May 27, 1998