Deterministic Routing on the Array with Reconfigurable Optical Buses

Sanguthevar Rajasekaran and Sartaj Sahni

Parallel Processing Letters, Vol. 7, No. 3, 1997, pp. 219-224

1. Introduction

Lemma    Let L be a LAROB of size n.  Consider a routing problem where O(1) packets originate from any node and O(1) packets are destined for any node.  This problem can also be solved in O(1) cycles.

2. Matrix Transpose Routing

Switch connection for any processor (i, j) :

SE			if i + j is odd

NW			if i + j is even

The matrix transpose permutation is a BPC permutation characterized by the vector (p/2–1, p/2–2, …, 1, 0, p, p–1, …, p/2).

Lemma    Matrix transpose can be performed on an n  n AROB in O(1) cycles.

3. BPC Permutation Routing

BPC: Bit Permute and Complement

Example:

N = 8, permutation  = (-2, 0, 1)

(a2, a1, a0)		(a2, a0, a1)

(1, 1, 0)			(0, 0, 1)

The basic idea is to decompose any BPC permutation into a sequence of five permutations.

Algorithm: (q is the packet.)

Phase I:		A = A1, A0

A1 = ai1, ai2, …, aik  and   A0 = aj1, aj2, …, ajk

Route the packet from A to A = A1 – A1, A1, A0

(If ai A1 and i is negative, then ai will appear in A as ai.)

# Phase II:	Route the packet form current location to

A1 – A1, A1, A0 – A0, A0

Phase III:	Partition the mesh into submeshes of size 2k  2k and employ matrix transpose within the submeshes. q will reach

A1 – A1, A0, A0 – A0, A1

Phase IV:	Routing is done along the columns, q will be in its destination row.

Phase V:	Finally, a routing step along the rows takes q to its desired destination.

Example:

N = 28, BPC permutation  = (6, -3, -4, 1, 0, -2, 5, 7)

Packet q originates from (a7, a6, a5, a4, a3, a2, a1, a0)

A1 = a7, a6, a5, a4 		A0 = a3, a2, a1, a0

A1 = a6, a4				A0 = a1, a0

Origin:		a7, a6, a5, a4, a3, a2, a1, a0

Phase I:		a7, a5, a6, a4, a3, a2, a1, a0

A1–A1, A1,     A0

Phase II:	a7, a5, a6, a4, a3, a2, a1, a0

A1–A1, 	A1,  A0–A0, A0

Phase III:	a7, a5, a1, a0, a3, a2, a6, a4

A1–A1, A0,  A0–A0, A1

Phase IV:	a0, a7, a1, a5, a3, a2, a6, a4

Phase V:	a0, a7, a1, a5, a6, a2, a4, a3

Case I 						Case II

Origin		(0, 0)   (00000000)			(9, 6)   (10010110)

Phase I		(6, 0)						(15, 6)

Phase II		(6, 4)						(15, 2)

Phase III	(4, 6)						(14, 3)

Phase IV	(1, 6)						(7, 3)

Phase V		(1, 12)  (00011100)			(7, 10)  (01111010)

Created by: Kuo-Shii Huang

Date: Nov. 6, 1999